EEG Setup

Preparation

- Choose cap size based on head circumference (if head circumference is in between the size of two caps, use the smaller one) => ask participants to send their head circumference when arranging the appointment (see TimeslotConfirmation_template for example)

- Add yellow and green electrodes to respective electrode-holes in the cap (Do not add reference & ground electrode yet)

  - Attention: one of the currently used caps (July 2023) has two yellow 9’s, one of them replacing the yellow 6. Please beware of correctly applying the electrodes!

- Fill 3-4 syringes with electrode gel and put needle on top

- Unplug all plugs from electricity line (amplifier, battery charger)

With Participant

- Measure midpoint on participant head (Electrode Cz) and mark it with red pen

  - Midpoint between nasion (depression between eyes) and inion (hill on back of head)

  - Midpoint between both ears (just behind the earlobe)

- Put cap on participant head with ground electrode (Cz) on midpoint

  - Double-check visually by having all central electrodes actually centrally aligned

  - Measure again if Cz is located in the midpoint

  - !! The wholes on the side should surround participants’ ears; close cap underneath the chin

- Clip splitter box on subjects’ shirt

- Enter ground and reference electrode on participant head

- Enter electrodes into Control box (Ground, Reference & splitter box electrodes)

- Enter USB-computer-cable into control box & turn it on

- Open program “ActiCapControl” on experiment computer

  - Click on ‘Impedance Check’ and ‘Impedance On’

  - Click on impedance on (good impedance: 20 kOhm; bad impedance 30-40 kOhm)

  - Timeout: 50 min

- Enter gel to all electrodes until all impedances are “good”

- Save impedances to computer

- Close “ActiCapControl”, turn off control box and detach it from USB cable

- Enter batteries into control box

- Enter amplifier endings into control box and turn it on

- Tell participants to only respond with right index finger! Move as little as possible!

  - Remind participants after practice trials (watch if they do it correctly during practice trials)

  - Tell them to shout if they have questions in between the blocks

EEG-Computer

- Turn on EEG computer and open “BrainVisionRecorder” program

- Open file “MSA_EEG_Maren”  settings are correctly saved already

- Click on the eye-symbol to see the EEG signal from all channels  check whether signal is smooth

- Click on Play to start recording (before starting the experiment)

  - If one electrode shows unusual amplitudes, try to solve it by adding more gel / adapting the gel at the corresponding electrode

  - To see which electrode is not working (Name of electrode corresponding to number on head), see DINA4 paper next to participant

  - While recording, check whether triggers are sent

- After the experiment, click the red square to stop recording

- The data file is automatically saved

- Close the experiment before stopping eeg recording

EEG setup after experiment is finished

- Unplug all plugs from control box

- Recharge batteries

- Take off EEG cap and clean with brush and clear water in bowl

  - !!!! Take care that no water comes to the splitter box

  - !!!! Take care only to brush the electrodes from the top -> be careful

- Wash EEG cap  if two sessions take place in a row, dry cap with fan available in the lab

- Sort & dry electrodes on the towel

- Give participants opportunity to wash their head (towel, shampoo), offer fan

- Compensate participants

  - VPn or Cash (1 credit per hour or 9 Euro per hour)

  - In addition, pay out money earned during experiment (not down amount presented to participants at the end of the task OR, if the task is ended, go to “feedbacklog” in the matlab workspace and look at variable “final payout” (all -1 values are invalid, the correct value is the one from the last row/column)